LED

T8一体化灯管规格书

T8 Incorporate LED Tube

| 0.6M | 1.2M | 1.5M |
| --- | --- | --- |
| T8-D0620 | T8-D1220 | T8-D1520 |
| 10W | 20W | 25W |

ProductAppearance Dimensions:

| T8 Incorporate Tube Datasheet | Product Features: |
| --- | --- |
|  | 1.With internal Non-isolated power supply. 2.Use the most mature international well-known LED SMD2835 as the light source, which can ensure the led tube brightness and long lifetime, also can reduce the light decay. 3.Use the high quality aluminum PCB for LED’s better heat dissipation. 4.High-grade PC cover with nice light transmittance (>95%), Transparent PC cover and frosted PC cover are available. 5.140º beam angle makes the lighting effect better. 6.Ambient temperature:-20℃~40℃, surface temperature: 42℃~45℃. 7.No mercury, lead or other hazardous materials; no radiation, no electromagnetic radiation, no frequency flash, instant startup, no hurting eyes. 8. IP rate: IP 20 |

| Standard Length | Dimension(mm) |  |  |  |
| --- | --- | --- | --- | --- |
|  | A | B | C | D |
| 0.6M | 600 | 33 | 31 | 28 |
| 1.2M | 1200 | 33 | 31 | 28 |
| 1.5M | 1500 | 33 | 31 | 28 |

Product Spec.

| Model number 型号 | Dimension 尺寸(mm) | CCT 色温 | Power(W) 功率 | Luminous Flum 光通量（lm） | Input Voltage 输入电压 | PF | LED Q,ty LED数量 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T8-D0620 | 600*28*33 | WW:2500-2800K 2800-3200K DW:3750-4250K PW: 4750-5300k 5700-6500k | 10W±8％ | 1300lm | AC95-265V | 0.92 | 120pcs |
| T8-D1220 | 1200*28*33 |  | 20W±8％ | 2600lm |  |  | 240pcs |
| T8-D1520 | 1500*28*33 |  | 25W±8％ | 3250lm |  |  | 288pcs |

Above test parameter has a tolerance ±8%

Photoelectric Test Report

10W 0.6M

20W 1.2M

25W 1.5M

Luminous Intensity Distribution:

Product Structure:

Installation:

Max.Connection Q’ty:12Pcs for 25W; 15Pcs for 20W; 30Pcs for 10W;(AC220V)

Attention

Please cut off the power before installation.

Please cut off the power if change lamp.

No touching the exposed power connection pins while the lamp is on power

Packing Info:

| Model | NW/GW （PC） | Carton Size | Q'ty/CTN | GW./ CTN |
| --- | --- | --- | --- | --- |
| T8-D0620 | 0.22kg | 670*220*220MM | 25PCS/Carton | 6.5kg |
| T8-D1220 | 0.41kg | 1270*220*220MM | 25PCS/Carton | 11.8kg |
| T8-D1520 | 0.51kg | 1570*220*220MM | 25PCS/Carton | 14kg |

Application:

Indoor Lighting / Schools / Universities / Hospital / Conference Rooms / Show Rooms / Factories / Offices / Chain Stores / Supermarkets / Metro Station / Underground Parking.